TGIIC — e-Tender for Biometric Facial Recognition Attendance System

# GOVERNMENT OF TELANGANA

# TELANGANA INDUSTRIAL INFRASTRUCTURE CORPORATION LIMITED (TGIIC)

# e-TENDER DOCUMENT

FOR

Supply, Installation, Commissioning and Comprehensive Maintenance of AI-based Biometric Facial Recognition Attendance System for TGIIC Head Office, all Zonal Offices and IALA Office, including Web-based Software, Dashboard and Cloud Hosting Services

e-Tender No.: TGIIC-MIS/34/2026-MIS-HO Date: [05-09-2026]

Issued by:

Telangana Industrial Infrastructure Corporation Limited (TGIIC) Head Office, Hyderabad, Telangana

Email: cio-iic@telangana.gov.in

TGIIC — e-Tender for Biometric Facial Recognition Attendance System

Table of Contents

Section I — Invitation for e-Tender Section II — Instructions to Bidders Section III — Scope of Work and ToR

Section IV — Bill of Quantities (BoQ) / Price Schedule Section V — Technical Specifications and Requirements Section VI — Service Level Agreement (SLA)

Section VII — General Conditions of Contract (GCC) Section VIII — Special Conditions of Contract (SCC) Section IX — Bid Evaluation Criteria

Section X — Bid Submission Requirement Section XI — Annexures (I – VIII)

Section XII — Fair Wages Clause and Statutory Compliance Bidder's Acceptance and Declaration

TGIIC — e-Tender for Biometric Facial Recognition Attendance System

Section I — Invitation to e-Tender

Telangana Industrial Infrastructure Corporation Limited (TGIIC), Government of Telangana, invites e-tenders (online) from eligible, reputed and established firms/companies having relevant experience, for “Supply, delivery, installation, testing, commissioning and comprehensive 3-year warranty/AMC maintenance of AI-based Biometric Facial Recognition Attendance Devices at TGIIC Head Office and all Zonal Offices, together with a Mobile Application for office & field-staff attendance, a Web-based Attendance Management Software with dashboard/MIS and integration support, and associated cloud hosting services, on turnkey basis including transportation and transit risk up to destination, complete”.

The detailed Terms and Conditions of this tender can be obtained from the Government of Telangana e-Procurement website https://eprocurement.telangana.gov.in. Prospective bidders should download the complete tender documents only from this website up to the last date of submission. Any Addendum/Corrigendum will be uploaded on the same website; bidders are advised to check it regularly. Bidders should study the complete bid document, eligibility criteria and scope of work carefully before submitting their online bid.

| Particulars | Details |
| --- | --- |
| e-Tender No. | TGIIC-MIS/34/2026-MIS-HO |
| Name of Work | Supply, installation, commissioning and 3-year comprehensive maintenance of AI-based Biometric Facial Recognition Attendance Devices with Mobile App for office & field staff, Web-based Attendance Software, Dashboard and Cloud Hosting, for TGIIC Head, 9 Zonal & IALA Offices |
| Total number of fixed devices | 34 (4 at Head Office + 2 at each of 9 Zonal Offices and 12 IALA offices) along with mobile app |
| EMD (Earnest Money Deposit) | Rs.12,000, payable online |
| Start Date for download of Bid document | 07-09-2026 @03:00 PM |
| Pre-bid meeting date & time | 11-09-2026 @11 AM |
| Last date & time for submission of bids | 23-09-2026 @ 3:00 PM |
| Date & time of Opening of technical bid | 23-09-2026 @3.30 PM |
| Period of contract | 36 months (3 years) from the date of Go-Live, extendable by mutual consent |
| Bid validity | 90 days from the date of opening of the technical bid |
| Bid processing fee | Rs.5900 vide Demand Draft in favour of TSIIC Ltd Hyderabad |

Issuer Address for Bid Submission and Correspondence: Chief Information Officer (CIO), TGIIC, Head Office, Hyderabad, Telangana. Email: cio-iic@telangana.gov.in

# Section II — Instructions to Bidders

- General

  - Important Dates — see the table in Section I above; the authoritative dates are those published on the e-Procurement portal for this e-Tender number.

  - Amendment to the Bid Document — At any time prior to the bid due date, TGIIC may, on its own initiative or in response to a bidder's request for clarification, amend the bid document by issuing a corrigendum on the e-Procurement website. TGIIC may, at its discretion, extend the bid due date to give bidders reasonable time to account for the amendment.

  - Important Timelines of the Project

| Phase | Scope | Timeline | Remarks |
| --- | --- | --- | --- |
| Phase I | Supply, installation, commissioning and 3-year comprehensive maintenance of AI-based Biometric Facial Recognition Attendance Devices with Mobile App for office & field staff, Web-based Attendance Software, Dashboard and Cloud Hosting, for TGIIC Head, 9 Zonal & IALA Offices | Within 3 weeks from the date of the Letter of Award (LOA) | All locations as directed by TGIIC officials |
| Phase II | Comprehensive warranty/AMC of devices, GSM/4G modules, software application and mobile app; hosting; support. | Within 3 weeks from the date of Letter of Award (LOA) With AMC for 36 months from the date of Go-Live at each location | As per SLA in Section 7 |

# Eligibility Criteria

To qualify for award of the contract, each bidder shall fulfil the following criteria

| S.No. | Eligibility Criterion | Supporting Document / Reference | Status |
| --- | --- | --- | --- |
| 1 | Valid Firm/Manufacturer Registration (Udyam/MSME/NSIC or equivalent) issued by the Central/State Government. | — | Mandatory |
| 2 | Copy of Permanent Account Number (PAN). | — | Mandatory |
| 3 | Copy of valid GST Registration Certificate. | — | Mandatory |
| 4 | EMD as specified in the e-Tender notice, paid online. | — | Mandatory |
| 5 | Affidavit on non-judicial stamp paper of Rs. 100 declaring no blacklisting/debarment by any | Annexure III | Mandatory |

| S.No. | Eligibility Criterion | Supporting Document / Reference | Status |
| --- | --- | --- | --- |
|  | Government department, PSU or autonomous body. |  |  |
| 6 | Statement of works on hand. | Annexure IV | Mandatory |
| 7 | Record of arbitration & litigation, if any, for the last 6 years. | Annexure I | Mandatory |
| 8 | Record of criminal cases, if any, against the firm/its partners/directors. | Annexure II | Mandatory |
| 9 | Minimum average annual turnover of Rs. 20 lakh in any one of the last 5 financial years, certified by a Chartered Accountant. | Annexure VII | Mandatory |
| 10 | Solvency Certificate for not less than Rs. 20 lakh from a Scheduled/Nationalised Bank, valid within 1 year of issue, OR equivalent liquid assets certified by a CA. | — | Mandatory |
| 11 | Experience of having successfully executed at least one similar contract (supply, installation and maintenance of AI/biometric face-recognition attendance devices with a web/mobile software application) of a minimum of 25 devices in any one financial year during the last 6 years, in any State/Central Government department, PSU, or reputed private/corporate client, supported by a completion/performance certificate. | — | Mandatory |
| 12 | Experience of having executed similar work of a minimum value of Rs. 6 lakh in any one financial year during the last 5 years. | — | Mandatory |
| 13 | ISO 9001:2015 (or equivalent quality) certification for the bidder or its OEM. | — | Mandatory |
| 14 | Manufacturer's Authorisation Form / OEM Authorised Reseller-Distributor Certificate for the biometric devices quoted. | — | Mandatory |
| 15 | Bidder information in Form P-I. | Annexure V | Mandatory |
| 16 | Certificate of conformity to the technical specifications, item-by-item. | Section V | Mandatory |
| 17 | The bidder, or any of its partners/directors, should not be currently blacklisted or debarred by any Government of India / Government of | — | Mandatory |

| S.No. | Eligibility Criterion | Supporting Document / Reference | Status |
| --- | --- | --- | --- |
|  | Telangana department, PSU, or autonomous body. |  |  |

The tender document is not transferable. Conditional bids are not acceptable and are liable for rejection.

# Cost of Tendering

The bidder shall bear all costs associated with the preparation and submission of its bid. TGIIC will not be responsible for these costs, regardless of the outcome of the tendering process.

# Site Visit

Bidders are encouraged, at their own cost, to visit the TGIIC Head Office and the Zonal & IALA Office locations listed in Section 4 to familiarise themselves with site conditions (network availability, power points, wall/desk space) relevant to installation, before submitting their bid. Failure to do so will not relieve the successful bidder of its obligations under the contract.

# Tendering Documents

  - The bid document consists of: Section I – Invitation to e-Tender; Section II – Instructions to Bidders; Section III – Scope of Work; Section IV – Bill of Quantities / Price Schedule; Section V – Technical Specifications; Section VI – Service Level Agreement; Section VII – General Conditions of Contract; Section VIII – Special Conditions; Section IX – Eligibility & Bid Documents Checklist; Section X – Annexures; Section XI – Fair Wages Clause.

  - Clarification of Tender Documents — A prospective bidder requiring clarification may submit written queries, in the format Section No. / Clause No. / Reference-Subject / Clarification Sought, to the address in Section I, by the date specified in the key dates table. TGIIC's response (without identifying the source) will be published on the e-Procurement website.

  - Amendment of Tender Documents — TGIIC may, at any time prior to the bid due date, modify the tender document by issuing a written Addendum on the e-Procurement website, and may extend the bid due date accordingly.

# Preparation & Submission of Tenders

  - Language — The bid and all correspondence shall be in English.

  - Documents Comprising the Tender — The bidder shall submit the Technical Bid and Financial Bid online through https://eprocurement.telangana.gov.in, comprising all documents listed at Clause

2.4 above, the priced BOQ (Section IV), and an undertaking of unconditional compliance with the technical specifications (Section V). GST shall be quoted extra, as applicable. Prices quoted shall remain fixed for the entire contract period and shall not be subject to variation.

  - Bid Due Date — Technical and Financial bids shall be submitted online on or before the date and time specified in the e-Tender notice.

  - Procedure for e-Tendering — Bidders shall register on https://eprocurement.telangana.gov.in, obtain a Class-III Digital Signature Certificate, and submit bids online only; offline/e-mail/fax bids will not be entertained. Scanned copies of all supporting documents shall be uploaded in PDF/JPEG format. The successful bidder shall submit original hard copies of all uploaded documents to the address in Section I within 7 days of the Letter of Award.

  - Bid Validity — Bids shall remain valid for 90 days from the date of opening of the technical bid. TGIIC may request an extension of validity in exceptional circumstances.

  - Opening of Bids — Technical bids will be opened online on the notified date. Financial bids of technically qualified bidders only will be opened subsequently, after completion of technical evaluation.

  - Rejection of Bids — TGIIC reserves the right to reject any or all bids and to annul the bidding process at any time, without assigning reasons and without liability to the affected bidders.

  - Confidentiality — Information relating to examination, clarification, evaluation and comparison of bids will be treated as confidential and not disclosed to bidders or any party not officially concerned with the process, until award of contract.

  - Earnest Money Deposit (EMD) — Each bid must be accompanied online by an EMD through the e-Procurement platform, or an equivalent Bank Guarantee valid for a minimum of 135 days (90 + 45) from the bid due date. Bids not accompanied by the requisite EMD will be summarily rejected. EMD of unsuccessful bidders will be released after issue of the Letter of Award to the successful bidder. The successful bidder's EMD will be retained as security until the Performance Security is furnished; it will be forfeited if the bidder withdraws its bid during the validity period, or fails to sign the contract/furnish Performance Security within 7 days of the Letter of Award.

  - Technical Rejection Criteria — A bid will be rejected if: it is not unconditionally valid for the required period; contains materially incorrect or misleading information; is received after the due date/time; is not digitally signed; does not cover the complete scope of work; does not conform to the mandatory technical specifications; or attempts to influence the evaluation process.

# Tender Opening and Evaluation

  - Technical Evaluation — A tender evaluation committee constituted by TGIIC will examine bids for responsiveness to the eligibility criteria (Clause 2.4) and the technical specifications (Section V), including verification of documents, OEM authorisation, and (if required) a demonstration of the proposed device/software. The committee's decision is final and binding.

  - Financial Evaluation — Financial bids of only the technically qualified bidders will be opened. The bidder quoting the lowest total landed cost (L1), evaluated on the total of the priced BOQ in Section IV inclusive of all one-time and 3-year recurring costs (hosting, API/support, AMC) but exclusive of GST, will be recommended for award, subject to compliance with all other conditions.

  - Clarification of Tenders — TGIIC may seek written clarification from any bidder during evaluation; no change in price or substance of the bid will be permitted in response.

# Award of Contract

  - Award — TGIIC will award the contract to the substantially responsive bidder offering the lowest evaluated total cost (L1), subject to satisfactory verification of eligibility and technical compliance. Notification of Award (Letter of Award / Work Order) will be issued in writing.

  - Performance Security — The successful bidder shall furnish an irrevocable Performance Bank Guarantee (PBG) of 5% of the total contract value within 7 days of the Letter of Award, valid for the contract period plus 3 months beyond the last day of the warranty/AMC period (i.e., valid for approximately 39 months). TGIIC may invoke the PBG for non-performance at any stage.

  - Signing of Agreement — The successful bidder shall execute the Contract Agreement on appropriate stamp paper within 7 calendar days of receipt of the Letter of Award, failing which the EMD may be forfeited and the bidder disqualified from future TGIIC tenders.

  - Jurisdiction — All disputes shall be subject to the jurisdiction of the competent courts at Hyderabad, Telangana only.

  - TGIIC's Right to Vary Scope — TGIIC may, by written order, increase or decrease the quantity of devices/locations by up to 30% of the original BOQ quantities at the same unit rates, without a fresh tendering process, before completion of deliveries.

# Section III — Scope of Work

TGIIC intends to deploy a unified AI-based Biometric Facial Recognition Attendance System across its Head Office and all Zonal Offices, supplemented by a Mobile Application enabling field staff (who do not report to a fixed office location) to mark attendance from the field, all integrated into a single Web-based Attendance Management Software with a central dashboard, MIS reporting, and cloud hosting. The successful bidder shall be responsible for end-to-end supply, installation, integration, training, and 3-year comprehensive warranty/AMC support.

# Location-wise Device Deployment

| S.No. | Location | No. of Biometric Devices- Zonal office | No. of Biometric Devices- IALA office |
| --- | --- | --- | --- |
| 1 | TGIIC Head Office, Hyderabad | 4 |  |
| 2 | Cyberabad Zonal Office | 2 | 2 |
| 3 | Karimnagar Zonal Office | 2 |  |
| 4 | Yadadri Zonal Office | 2 |  |
| 5 | Khammam Zonal Office | 2 |  |
| 6 | Medchal–Siddipet Zonal Office | 2 | 10 |
| 7 | Patancheru Zonal Office | 2 |  |
| 8 | Shamshabad Zonal Office | 2 |  |
| 9 | Nizamabad Zonal Office | 2 |  |

| S.No. | Location | No. of Biometric Devices- Zonal office | No. of Biometric Devices- IALA office |
| --- | --- | --- | --- |
| 10 | Warangal Zonal Office | 2 |  |
|  | Total | 22 | 12 |

Total fixed biometric devices required: 44 nos. (4 at Head Office + 2 each at 9 Zonal Offices+12 IALA offices).

Note: the agency needs to connect the machines to the nearest LAN/Wi-Fi available at each site listed above

# A Office-wise Employee Strength (for Enrolment & Licence Sizing)

The table below gives the existing employee strength at TGIIC Head Office and each Zonal Office, by category, as the basis for facial-template enrolment, device user-capacity sizing (Section V, Clause 5.1) and the per-enrolled-employee monthly support cost quoted in Section IV, Part B.

| S.No. | Office / Location | Regular | Outsourcing | Contract | Consolidated Pay | IALA | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TGIIC Head Office, Hyderabad | 32 | 101 | 3 | 22 | 0 | 158 |
| 2 | Cyberabad Zonal Office | 5 | 21 | 1 | 1 | 13 | 41 |
| 3 | Karimnagar Zonal Office | 3 | 13 | 2 | 0 | 0 | 18 |
| 4 | Yadadri Zonal Office | 3 | 11 | 0 | 0 | 11 | 25 |
| 5 | Khammam Zonal Office | 2 | 9 | 0 | 0 | 0 | 11 |
| 6 | Medchal–Siddipet Zonal Office | 9 | 40 | 3 | 0 | 105 | 157 |
| 7 | Patancheru Zonal Office | 7 | 33 | 3 | 1 | 0 | 44 |
| 8 | Shamshabad Zonal Office | 7 | 69 | 2 | 2 | 28 | 108 |
| 9 | Nizamabad Zonal Office | 2 | 9 | 0 | 0 | 0 | 11 |
| 10 | Warangal Zonal Office | 4 | 18 | 1 | 0 | 4 | 27 |

| S.No. | Office / Location | Regular | Outsourcing | Contract | Consolidated Pay | IALA | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | 74 | 324 | 15 | 26 | 161 | 600 |

Total employees to be enrolled across all locations: 600 (74 Regular + 324 Outsourcing + 15 Contract

+ 26 Consolidated Pay + 161 IALA).

# Biometric Facial Recognition Attendance Devices (Head Office & Zonal Offices)

Scope:

    - Supply, delivery, installation and commissioning of AI-based Face-Recognition biometric attendance devices as per the technical specifications in Section V, at each location listed above.

    - Site survey prior to installation, to finalise mounting location for ease of employee access, power availability and network connectivity.

    - Supply and installation of ABS anti-vandal protective casing with lock at each device location.

    - Local electrical point (15A/6A switch board as required) and network (CAT6/LAN) connectivity from the nearest switch/router to the device, including cabling, conduiting and testing, at each location — actual cable lengths to be assessed during the site survey and billed against the BOQ provisional quantities in Section IV.

    - Enrolment of all Head Office, Zonal & IALA Office employees' facial templates and master data (name, designation, employee ID, department, photo, etc.) into the devices and central software.

    - Testing and commissioning of every device, with a written Go-Live/commissioning certificate issued location-wise.

    - On-site training to designated TGIIC staff/administrators at each location on device operation and basic troubleshooting.

    - Three (3) years comprehensive on-site warranty/AMC, covering all hardware components (including camera module, display, battery/power unit, and 4G/GSM module where supplied)

— see Section VI for SLA.

# Mobile Application Staff Attendance

Scope:

    - An Android and iOS mobile application enabling office staff & field staff who do not operate from a fixed TGIIC office to mark attendance using facial recognition combined with GPS geo-tagging, from their registered smartphones.

    - Online self-registration/enrolment of field staff facial templates through the mobile app, subject to admin approval.

    - Real-time push of field-staff attendance punches (with photo, GPS coordinates and timestamp) to the same central Web-based Attendance Software used for office locations, so that Head Office and Zonal Office attendance and field-staff attendance are visible on one unified dashboard such filed staff attendance should be approved by concerned reporting officer.

    - Employee Self-Service (ESS) features on the mobile app: view attendance/leave history, apply for leave/tour, manual attendance request with approval workflow, and push notifications.

    - Support for the number of field-staff users indicated by TGIIC — [field-staff headcount to be confirmed by TGIIC] — with the software licensed/priced on a per-user basis as quoted in Section

IV, and scalable to TGIIC's actual field workforce without re-tendering, subject to the 30% variation clause at Section II, Clause 2.27.

# Web-based Attendance Management Software, Dashboard and Integration

    - Development/configuration and implementation of a single Web-based Attendance Management Software (licence to TGIIC) covering all Head Office, Zonal Office, IALA Office and mobile-app (for all staff) attendance data on one central server/cloud instance.

    - Admin panel and role-based dashboards for Head Office and each Zonal Office, with drill-down by location, department and employee category (regular, contractual, field staff).

    - Daily / monthly / yearly attendance, leave, late-arrival, early-departure, overtime, and absentee reports, with export and scheduled e-mail reports to designated officers.

    - Real-time device-status monitoring and alerts (device offline, tamper, low storage, etc.).

    - API/database-level integration with TGIIC's existing HRMS/payroll system (details of the existing system, if any, to be shared by TGIIC during the implementation phase) for employee master synchronisation and attendance data export.

    - Data backup, role-based access control, and security-audit compliance; admin credentials and full documentation before final payment; all data remain the exclusive property of TGIIC.

    - Dedicated Executive / Higher-Officials Dashboard, available both on the web portal and within the mobile app, giving senior TGIIC management (e.g., MD/CEO, CIO, Zonal/Department Heads) a real-time, role-based, read-only view of organisation-wide attendance across Head Office, all Zonal Offices and field staff — see Section V, Clause 5.4 for detailed features.

# Hosting

    - The bidder shall host the Web-based Attendance Software and Mobile Application backend on a secure cloud environment, including domain name, SSL certificate and annual hosting/renewal charges, quoted separately as a recurring annual cost in Section IV for the 3-year contract period.

    - The bidder shall provide TGIIC with read access/reporting credentials and ensure data-residency/security compliant with Government of Telangana IT policy; TGIIC shall retain full ownership of all hosted data at all times, including on contract exit (data-export/handover obligations at Section VII, Clause 7.9).

# Implementation, Training and Support

    - Detailed project plan and site-wise implementation schedule within 1 weeks of the Letter of Award.

    - Training to TGIIC IT/HR staff (Head Office) and designated coordinators at each Zonal Office, on software administration, report generation and first-level troubleshooting.

    - A single point of contact (helpdesk) for logging and tracking all hardware and software issues across all locations, throughout the 3-year warranty/AMC period, as per the SLA in Section VI.

TGIIC — e-Tender for Biometric Facial Recognition Attendance System

Section IV — Bill of Quantities / Price Schedule

Bidders shall quote in the format below. Quantities marked “provisional” are indicative and shall be finally billed on the basis of actual work executed / users enrolled, verified jointly with TGIIC, at the unit rates quoted here (Clause 7.10, Section VII). All prices shall be quoted in Indian Rupees, exclusive of GST, and shall remain firm for the entire contract period. The bidder quoting the lowest Grand Total (Part A + Part B) shall be treated as L1 for award, subject to technical qualification.

# Part A — One-Time (Capital) Cost

| S.No. | Item Description | Unit | Qty | Unit Rate (Rs.) | Total (Rs.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Supply, installation, testing & commissioning of AI-based Biometric Facial Recognition Attendance Device, complete with ABS anti-vandal protective casing with lock, as per Section V specifications | No. | 34 |  |  |
| 2 | CAT 6 LAN cabling, conduiting and termination from nearest switch/router to device | Rmt | 1 |  |  |
| 3 | 3-core 1 sq.mm electrical cabling with 15A/6A switch board setup per device location | No. | 34 |  |  |
| 4 | Web-based Attendance Management Software with Admin Dashboard, MIS reports, HRMS/Payroll integration and a dedicated Executive / Higher-Officials Dashboard (web) — with three years licence to TGIIC | Lumpsum | 1 |  |  |
| 5 | Mobile Application (Android & iOS) for Field-Staff attendance — facial recognition + GPS, ESS features — including the Executive / Higher-Officials Dashboard view for senior officials — licensed for all users | Lumpsum | 1 |  |  |
| 6 | Implementation: configuration, employee master/data migration, enrolment (Head Office + Zonal Offices + IALA officers + field staff), integration, testing and training | Lumpsum | 1 |  |  |

# Sub-Total, Part A (Rs.):

TGIIC — e-Tender for Biometric Facial Recognition Attendance System

# Part B — Recurring Cost (3-Year Contract Period)

| S.No. | Item Description | Unit | Qty | Unit Rate (Rs.) | Total (Rs.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Comprehensive on-site Warranty / AMC of all devices, casings (preventive + breakdown maintenance) — included in Part A device cost; no separate recurring charge unless quoted | Year | 3 |  |  |
| 2 | Annual cloud hosting charges (server, domain name registration/renewal, SSL certificate) for the Web-based Software and Mobile App backend with security audit certificate from cert in empanelled agency. | Year | 3 |  |  |
| 3 | Monthly face-matching API, software support and maintenance, per enrolled employee (Head Office + Zonal Office + IALA offices +field staff) | Per employee / month | No of employees × 36 months |  |  |

Sub-Total, Part B (Rs.):

GRAND TOTAL (Part A + Part B), exclusive of GST (Rs.):

GST @ 	% extra (Rs.):

TOTAL BID PRICE, inclusive of GST (Rs.):

Total Bid Price in words:

# Payment Terms

- 80% of Part A cost shall be released location-wise, on successful installation, commissioning and Go-Live certification of the devices/software at that location, against submission of invoice, delivery challan, installation report and warranty certificate.

- Balance 20% of Part A cost shall be released in three instalments tied to satisfactory maintenance performance: 5% on completion of Year 1 (with maintenance report), 5% on completion of Year 2 (with maintenance report), and 10% on satisfactory completion of the full 3-year warranty/AMC period.

- Part B recurring charges (hosting and monthly API/support) shall be invoiced and paid quarterly (or annually, for hosting) in arrears, against a satisfactory-service certificate co-signed by the TGIIC location coordinator, subject to SLA compliance and penalty deductions as per Section VI.

- No advance payment shall be made. All payments shall be released electronically (NEFT/RTGS) to the bidder's registered bank account, subject to applicable TDS.

TGIIC — e-Tender for Biometric Facial Recognition Attendance System

Section V — Technical Specifications

Bidders shall furnish item-wise compliance (Yes/No) against every specification below, along with supporting technical/OEM datasheets. Any “No” or unsupported claim may render the bid technically non-responsive. Where a range or minimum is specified, the bidder's offer must meet or exceed it.

# Biometric Facial Recognition Attendance Device

| S.No. | Parameter |  |  | Minimum Required Specification |  |  |  |  |  | Compliance (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Identification modes |  |  | Face; optional Face + Card / |  |  |  |  |  |  |
| 2 | Face user capacity |  |  | Minimum 1,500 users, upgradable to 3,000+ |  |  |  |  |  |  |
| 3 | Identification speed |  |  | ≤ 0.3 seconds |  |  |  |  |  |  |
| 4 | Liveness / spoof detection |  |  | Mandatory (anti-photo/video spoofing) |  |  |  |  |  |  |
| 5 | Mask detection |  |  | Yes |  |  |  |  |  |  |
| 6 | Transaction log storage |  |  | Minimum 1,00,000 logs |  |  |  |  |  |  |
| 7 | Onboard storage |  |  | Minimum 8 GB EMMC flash (or higher) |  |  |  |  |  |  |
| 8 | Processor |  |  | Dual-core, 1 GHz or better |  |  |  |  |  |  |
| 9 | Operating system |  |  | Embedded Linux (or equivalent secure OS) |  |  |  |  |  |  |
| 10 | Camera |  |  | 2 MP dual (binocular) camera, 1080p / 30fps or higher, live-face detection |  |  |  |  |  |  |
| 11 | Display |  |  | Minimum 4.5″ IPS ≥1280×720 resolution |  |  | touchscreen, |  |  |  |
| 12 | Working light range |  |  | 0.1 – 20,000 LUX (mandatory) |  |  |  |  |  |  |
| 13 | Working distance |  |  | 0.3 m – 1.5 m or better |  |  |  |  |  |  |
| 14 | Operating humidity | temperature | / | -20°C to 70°C; 5%–90% humidity |  |  |  |  |  |  |
| 15 | Connectivity |  |  | TCP/IP LAN (10/100), built-in Wi-Fi, USB; optional built-in 4G with SIM slot |  |  |  |  |  |  |
| 16 | Data push |  |  | Real-time push to central server over LAN/Wi-Fi/4G (MQTT/HTTPS or equivalent) |  |  |  |  |  |  |
| 17 | Enrolment options |  |  | Through device, web application, and mobile app (all three) |  |  |  |  |  |  |
| 18 | Power input |  |  | 12V DC, datasheet) | ~1.5A | (or | as | per | OEM |  |
| 19 | MTBF |  |  | > 50,000 hours |  |  |  |  |  |  |

| S.No. | Parameter | Minimum Required Specification | Compliance (Y/N) |
| --- | --- | --- | --- |
| 20 | Protective casing | ABS anti-vandal, weather-resistant, lockable enclosure |  |
| 21 | Warranty | 3 years comprehensive on-site, including AMC |  |
| 22 | Spares & OEM support availability | Minimum 7 years from date of supply |  |

# Web-based Attendance Management Software

    - Configurable employee master with unlimited custom fields; support for multiple employee categories (regular, contractual, field staff, vendor staff, etc.) with category-wise policies.

    - Role-based access control (module-wise view/add/edit/delete) mapped to Circle/Zone/Department and user type; Active Directory / SSO integration where available at TGIIC.

    - Reports: department/location-wise daily, monthly and date-range attendance; exception, muster, who's-in, arrival, late-arrival, early-departure, overtime, absentee, and continuous-late/absent reports — each exportable and schedulable for automatic e-mail delivery.

    - Real-time dashboard with drill-down: device status, enrolled vs. un-enrolled employees, headcount by shift/department/designation, and live punch feed.

    - API/database-level integration with TGIIC's HRMS/payroll system for employee master sync and attendance/leave export.

    - Complete audit trail (user logs, device-command logs, notification logs, error logs) with export to Excel.

    - Data backup, disaster-recovery provisions, and compliance with applicable Government of Telangana / CERT-In data-security guidelines; software licence and full documentation to be handed over to TGIIC.

    - Shall Comply all norms as DPDPA act -2023

# Mobile Application

    - Android and iOS application for facial-recognition attendance with GPS geo-tagging at the time of punch.

    - Online enrolment/registration of field staff, subject to admin approval workflow.

    - Employee  Self-Service:  attendance/leave  history,  leave  and  tour  application,  manual-attendance request with approval, push notifications, password management.

    - Employee Self Service:-Attendance/Leave history, leave and tour application, manual attendance request with approval, push notifications, password management

    - Real-time synchronisation of field-staff punches with the central Web-based Attendance Software, viewable on the same dashboard as Head Office/Zonal Office data.

    - Executive Dashboard:-web portal and mobile app; role-based login for designated higher officials

    - Offline attendance: -In case of non-availability of the internet service, Mobile-App should take the Employee Punch (In & Out) and once the internet connection is restored the application should automatically synchronize the stored punch data with server, while retaining the actual date and time of the punch

    - Hosting: -Secure cloud environment and VAPT or equivalent security-audit certificate after go-live; remediation’s of findings

    - ISO 27701 Certification or any other equivalent certification

# Executive / Higher-Officials Dashboard (Web Portal & Mobile App)

    - A dedicated Executive Dashboard / Management Information view, accessible both from the web-based software and as a screen within the same mobile app, exclusively for designated higher officials (e.g., MD/CEO, CIO, Zonal/Department Heads, HR Head), secured by role-based login separate from regular employee/field-staff access.

    - Organisation-wide, real-time summary: present / absent / or-leave / late counts, consolidated across Head Office and all Zonal Offices, with one-tap drill-down from the overall TGIIC figure down to zone, department and individual employee.

    - Field-staff attendance and last-known punch location shown on a map view, for officials who need visibility of field operations.

    - Graphical trend analytics: day-wise, month-wise and zone-wise attendance/punctuality comparisons, late-arrival and absenteeism trends, and month-on-month performance of each Zonal Office.

    - Configurable real-time alerts and push notifications to higher officials for defined exceptions — e.g., unusually high absenteeism at a location, a device going offline, or a security/tamper flag.

    - One-tap export of an executive summary report (PDF/Excel) from either the web dashboard or the mobile app, and scheduled auto-delivery of this summary to higher officials by e-mail at a configurable frequency (daily/weekly/monthly).

    - The mobile-app version of this dashboard shall mirror the web dashboard's key metrics and drill-downs (not a stripped-down view), so that higher officials can review attendance status on the go without needing the web portal.

# General features required

| SN | features | Requirement |
| --- | --- | --- |
| 1 | Employee Login | Every employee should have individual login to see his/her attendance and leave details |
| 2 | Attendance Dashboard | Employee should see daily/monthly attendance, In-Time, Out-Time and status |
| 3 | Leave Balance | System should show CL, EL, Sick Leave and all applicable leave balances |
| 4 | Leave Availed | Employee should see how many leaves were availed and on which dates |
| 5 | Late Permission | Employee should see late permissions availed, pending and available, as per TGIIC rules |
| 6 | Online Leave Application | Employee should apply for leave through portal/mobile app |
| 7 | Approval System | Leave/permission should automatically move to Reporting Officer/Competent Authority for approval |

| 8 | Attendance Regularization | Employee should be able to request correction for missed/incorrect biometric punches |
| --- | --- | --- |
| 9 | Mobile App | Mobile app should allow employees to view attendance, leave balance, leave history and applications |
| 10 | Biometric Accuracy | Shall work accurately, speed, anti-spoofing/liveness and performance in different lighting conditions |
| 11 | Proxy Attendance | The system prevents one employee from marking attendance for another person |
| 12 | Network Failure | Attendance should be recorded even during network failure and automatically sync when connectivity returns |
| 13 | Centralized System | All Head Office, Zonal Offices and IALA offices should be connected to one central system |
| 14 | HR Integration | Employee master data should automatically reflect transfers, joining, retirement, designation changes etc. |
| 15 | Reports | HR should be able to generate attendance, leave, late-coming, permission and absenteeism reports |
| 16 | Role-Based Access | Employee, Reporting Officer, Zonal Office, HR and Administrator should have different access levels |
| 17 | Notifications | Employee should receive notification when leave/permission is approved, rejected or pending |
| 18 | Data Security | Ask how biometric and employee data will be encrypted, stored and protected |
| 19 | Data Ownership | All TGIIC employee/attendance/leave data should remain the property of TGIIC |
| 20 | Demonstration | Shall give a live demo of the complete system before making the application live |

# Section VI — Service Level Agreement (SLA)

This SLA defines the levels of service to be provided by the bidder to TGIIC for the duration of the contract (36 months from Go-Live at each location). TGIIC and the bidder shall jointly review SLA performance monthly.

# Key Terms

| Term | Definition |
| --- | --- |
| Delivery Timeline | Maximum allowable time for supply & delivery of devices after Letter of Award (indicatively 4 weeks, see Section III timeline). |
| Warranty / AMC Period | 3-year comprehensive on-site support including preventive and breakdown maintenance, from date of Go-Live at each location. |
| Response Time | Time to acknowledge a logged complaint — within 1 working hour of logging. |
| Resolution Time | Time to resolve the complaint after response — within 4 working hours. |

| Term | Definition |
| --- | --- |
| Preventive Maintenance | Scheduled maintenance visit once every 6 months during the warranty/AMC period. |
| Software/Portal Uptime | Web-based software and mobile-app backend to be available ≥ 99% of the time, measured monthly, excluding pre-notified maintenance windows. |

# SLA Matrix and Penalties

| Component | SLA Timeline | Penalty for Breach |
| --- | --- | --- |
| Delivery & installation of devices | Within the timeline in Section III | 2% of pending material cost for the first week of delay, plus 1.5% per additional week, capped at 5% of pending material cost |
| Complaint response | 1 working hour | Counted towards resolution-time SLA below |
| Complaint resolution (hardware fault, repair/replacement) | 4 working hours from response | Rs. 300 per hour of delay beyond 4 hours, capped at Rs. 3,000 per incident per device |
| Preventive maintenance visit | Once every 6 months | Written non-compliance warning; repeated default treated as breach under Section VII, Clause 7.13 |
| Software/portal uptime | ≥ 99% monthly | 0.5% of the monthly Part B invoice value for every 1% shortfall below 99%, capped at 5% of that month's invoice |
| Monthly MIS/SLA compliance report | By the 5th of the following month | 1% deduction from the monthly invoice, if not submitted on time |

Penalty formula for delivery delay: Penalty (%) = min [2 + 1.5 × (W − 1), 5], where W = number of weeks delayed beyond the agreed delivery timeline, applied to the value of pending (undelivered) material. Maximum penalty: 5% of pending material cost.

# Security & Data Terms

    - TGIIC shall retain ownership of all, databases, employee data and associated documentation at all times.

    - Any third-party libraries/components used shall be open-source or appropriately licensed, with no known security vulnerabilities.

    - On expiry/termination of the contract, the bidder shall hand over the application, software Licence, admin credentials, and complete documentation, and provide a full, verified export of all attendance/employee data to TGIIC or its nominated successor agency, free of cost, within 30 days.

# SLA Review Process

    - Either party may raise an SLA issue in writing, with a summary of the problem and proposed resolution.

    - A review meeting shall be held within 7 working days of the issue being raised (24 hours' notice for non-emergency issues).

    - Unresolved issues shall be escalated per the dispute-resolution procedure in Section VII, Clause 7.14.

# Section VII — General Conditions of Contract

  - Definitions

“Agreement”/“Contract” means the contract signed between TGIIC and the successful bidder, together with all annexures. “Bidder”/“Vendor” means the firm awarded the contract. “Go-Live”/“Commissioning Date” means the date on which a location is certified fully operational. “Warranty/AMC Period” means the 3-year comprehensive maintenance period from Go-Live. “Works” means all supply, installation, software, hosting and maintenance services under this contract. “Day” means calendar day unless stated as “working day”.

# Language and Governing Law

The contract shall be in English and governed by the laws of India. Disputes are subject to the jurisdiction of the competent courts at Hyderabad, Telangana.

# Notices

Any notice under the contract shall be deemed served when delivered by e-mail or registered post/courier to the addresses stated in the contract.

# Sub-Contracting

The bidder shall not sub-contract the whole or any material part of the works without TGIIC's prior written consent. Any permitted sub-contracting shall not relieve the bidder of its obligations or liabilities.

# Key Personnel

The bidder shall deploy experienced, qualified personnel (Annexure VIII) and maintain at least one point of contact reachable during business hours across the contract period. TGIIC may require replacement of any bidder personnel found unsuitable, with adequately qualified substitutes provided within 7 days.

# Bidder's Risk and Indemnity

The bidder shall be solely responsible for loss of or property damage, and injury to or death of personnel, arising from its performance of the contract, and shall indemnify TGIIC against all related claims, including any claim of infringement of intellectual property rights arising from the bidder's solution.

# Insurance

The bidder shall maintain adequate insurance cover (including for goods in transit and installed equipment) for the contract period, and furnish policy details to TGIIC on request.

# Intellectual Property and Ownership

All software, databases, process documentation, training material and data created or used in the performance of this contract shall be the exclusive property of TGIIC. The bidder shall not use TGIIC's data, branding or the deployed solution for any other client without TGIIC's prior written consent. Any technical know-how or third-party licensed components used shall be disclosed and appropriately licensed to TGIIC for the life of the solution.

# Confidentiality

Both parties shall keep confidential all documents, data and information exchanged under the contract, and shall not disclose the same to any third party without prior written consent, except as required by law. This obligation survives termination of the contract.

# Cost Control — Bill of Quantities

The BOQ in Section IV contains estimated/provisional quantities for certain items (cabling, 4G modules, employee-linked recurring items). Actual billing shall be based on quantities actually supplied/installed/enrolled and jointly verified with TGIIC, at the unit rates quoted, without requiring a fresh tender. Any variation beyond 30% of the original BOQ value shall require TGIIC's prior written approval.

# Taxes

Quoted prices shall be exclusive of GST, which shall be paid extra as applicable and shown separately in each invoice. All other duties/levies, if any, shall be included in the quoted price and shall not vary during the contract period.

# Time Control and Commissioning

The bidder shall commence work on receipt of the Letter of Award and complete supply, installation and commissioning within the timelines in Section III. TGIIC shall issue a location-wise commissioning/Go-Live certificate on successful completion. Reasonable extension of time may be granted for delays not attributable to the bidder, on written request within 14 days of the cause of delay.

# Termination

TGIIC may terminate the contract, with written notice, for breach including but not limited to: persistent failure to meet SLA timelines; supply of non-conforming/counterfeit equipment; sub-contracting without consent; corrupt or fraudulent practice; or insolvency of the bidder. On termination for bidder default, TGIIC may forfeit the Performance Security and procure the balance work from another agency at the bidder's risk and cost. TGIIC may also terminate for its own convenience, with payment for work satisfactorily completed up to the date of termination.

# Dispute Resolution

Any dispute shall first be referred in writing to the CIO, TGIIC, for resolution within 30 days. If unresolved, either party may refer the matter to arbitration under the Arbitration and Conciliation Act, 1996, with the seat of arbitration at Hyderabad, without prejudice to either party's right to approach a competent court at Hyderabad.

# Force Majeure

Neither party shall be liable for delay or failure to perform its obligations due to causes beyond its reasonable control (war, civil disturbance, epidemic, natural disaster, or government order). The affected party shall notify the other in writing within 30 days of occurrence and cessation of such event, and both parties shall use best efforts to mitigate the impact.

# Performance Security

As per Section II, Clause 2.24. The Performance Security shall be forfeited on breach of contract, and shall otherwise be released within 30 days of satisfactory completion of the contract period, including full data/asset handover under Clause 6.3.

# Section VIII — Special Conditions

    - Time is of the essence of this contract; the entire scope shall be completed within the timelines stipulated in Section III, from the date of the Letter of Award.

    - All equipment shall conform to relevant IS/BIS/IEC specifications and applicable electrical safety standards.

    - The bidder shall disclose the location(s) of its service centre(s) covering Hyderabad and the Zonal Office districts listed in Section III, and shall maintain adequate OEM spares for replacement in case of repair.

    - Camera module, display, battery/power unit and 4G/GSM module (where supplied) are covered under the 3-year warranty/AMC at no extra charge.

    - In the event of device replacement under warranty, the bidder shall migrate all enrolled facial templates and historical attendance data from the old device to the replacement device at no extra cost.

    - Manpower required for day-to-day support/troubleshooting across all locations is included within the AMC cost; no separate manpower charges shall be payable.

    - The bidder shall submit details of its technical support staff and an escalation matrix to TGIIC before commencement of the project, and shall promptly communicate any changes.

    - The bidder shall maintain a minimum of 10% of the total device count as standby units (with all spares) throughout the warranty/AMC period, for immediate replacement.

    - In case of physical damage, loss or theft of a device not attributable to a manufacturing defect, the bidder shall replace it promptly; TGIIC shall bear the cost of replacement, as mutually agreed, unless the loss is attributable to the bidder's negligence.

    - The bidder shall be responsible for enrolling new employees (and off-boarding exited employees) into the system throughout the contract period, at no extra cost, as part of the AMC.

    - The Web-based Attendance Software shall be customised for additional reports as reasonably required by TGIIC during the contract period, at no extra cost, provided such customisation does not materially alter the agreed scope.

    - The bidder shall provide a security-audit certificate (VAPT or equivalent) for the hosted software and mobile application after go-live, and shall remediate any findings from a TGIIC-commissioned security audit at no extra cost.

    - TGIIC may enhance or reduce the quantity of devices/locations/users by up to 30% of the BOQ quantities, at the same unit rates, without a fresh tendering process, as detailed in Section VII, Clause 7.10.

    - Penalties shall be levied strictly as per the SLA in Section VI; repeated non-compliance (3 or more breaches in any quarter) may be treated as a material breach under Section VII, Clause 7.13.

    - Complaints may be logged by TGIIC through phone, e-mail, SMS/WhatsApp, letter, or directly through the software/mobile app (a complaint-logging feature shall be built into the software).

# Section IX — Bid Documents Checklist

Bidders shall upload scanned copies of the following documents online, in the sequence below. Bids incomplete in any mandatory item is liable to summary rejection.

| S.No. | Document | Reference | Uploaded (Y/N) |
| --- | --- | --- | --- |
| 1 | Firm/Manufacturer Registration (Udyam/MSME/NSIC) | Section II, Cl. 2.4(1) |  |
| 2 | PAN Card copy | Section II, Cl. 2.4(2) |  |
| 3 | GST Registration Certificate | Section II, Cl. 2.4(3) |  |
| 4 | EMD (online payment receipt / BG) | Section II, Cl. 2.4(4) |  |
| 5 | Affidavit of non-blacklisting (stamp paper) | Annexure III |  |
| 6 | Statement of works on hand | Annexure IV |  |
| 7 | Record of arbitration & litigation | Annexure I |  |
| 8 | Record of criminal cases | Annexure II |  |
| 9 | CA-certified turnover statement (last 5 FYs) | Annexure VII |  |

| S.No. | Document | Reference | Uploaded (Y/N) |
| --- | --- | --- | --- |
| 10 | Solvency Certificate / liquid-assets certificate | Section II, Cl. 2.4(10) |  |
| 11 | Similar-work experience / completion certificates | Section II, Cl. 2.4(11)–(12) |  |
| 12 | ISO 9001:2015 (or equivalent) certificate | Section II, Cl. 2.4(13) |  |
| 13 | Manufacturer's Authorisation / OEM certificate | Section II, Cl. 2.4(14) |  |
| 14 | Bidder information — Form P-I | Annexure V |  |
| 15 | Technical compliance statement (item-wise) | Section V |  |
| 16 | Power of Attorney (if applicable) | on request |  |
| 17 | Priced Bill of Quantities / Financial Bid | Section IV |  |
| 18 | Undertaking of unconditional compliance | Section II, Cl. 2.11 |  |

# Section X — Annexures

Annexure I — Record of Arbitration and Litigation

The bidder shall chronologically record any disputes with previous clients in the last 10 years, indicating whether arbitration or litigation, nature, duration, and amount of claim involved.

| S.No. | Project & Client | Nature of Dispute | Period (From–To) | Amount Claimed (Rs. lakh) | Result & In Favour Of |
| --- | --- | --- | --- | --- | --- |

Name: 	 Designation: 	 Signature & Seal:

# Annexure II — History of Criminal Cases

The bidder shall furnish details of any criminal case registered against the firm or its partners/directors.

| S.No. | Police Station / Court | FIR No. & Date | Details of Case | Present Stage |
| --- | --- | --- | --- | --- |

(If none, state “NIL”.) Name: 	Designation:

Signature & Seal:

# Annexure III — Affidavit (on Non-Judicial Stamp Paper of Rs. 100)

(e-Tender No.: [•], dated: [•])

I/We, 	, son/daughter of 		, proprietor/partner/authorised signatory of M/s. 		, having its registered office at 		, do hereby solemnly affirm and declare as follows:

- That the firm/company is not currently blacklisted or debarred by any State/Central Government department, PSU, or autonomous body in India.

- That no criminal case is pending against the firm or its partners/directors save as disclosed in Annexure II.

- That all information and documents furnished along with this bid are true and correct to the best of my/our knowledge, and that I/We shall be liable for any consequences if found otherwise at any stage.

- That I/We have read and understood the terms and conditions of the tender document and unconditionally accept the same.

Deponent

Verified at 	on this    day of 	, 20 , that the contents of the above affidavit are true and correct to the best of my/our knowledge and belief.

# Annexure IV — Statement Showing Works on Hand

| S.No. | Name of Work | Client | Contract Value (Rs. lakh) | Date of Start | Stipulated Completion Date | % Completed |
| --- | --- | --- | --- | --- | --- | --- |

# Annexure V — Form P-I: Bidder Information

| Particulars | Details |
| --- | --- |
| Name of the firm/company |  |
| Registered address |  |
| Nature of firm (Proprietorship/Partnership/Pvt Ltd/LLP/etc.) |  |
| Year of establishment |  |
| PAN |  |
| GSTIN |  |
| Udyam/MSME/NSIC Registration No. |  |
| ISO Certification No. & validity |  |

| Particulars | Details |
| --- | --- |
| OEM(s) represented / authorised for |  |
| Name, designation & mobile no. of authorised signatory |  |
| E-mail for correspondence |  |
| Bank name, branch, account no. & IFSC |  |

# Annexure VI — Proforma of Performance Bank Guarantee (PBG)

(To be furnished on a Bank's letterhead / stamp paper, from a Scheduled/Nationalised Bank)

In consideration of Telangana Industrial Infrastructure Corporation Limited (TGIIC) having awarded to M/s. 	 (“the Bidder”) a contract for “Supply, installation, commissioning and 3-year comprehensive maintenance of AI-based Biometric Facial Recognition Attendance Devices with Mobile App, Web-based Software and Hosting” vide Letter of Award No.

dated 	, and the Bidder having agreed to furnish a Performance Bank Guarantee for Rs. 		 (Rupees 	 only), being 5% of the contract value, for the due performance of the said contract:

We, 		 Bank, hereby unconditionally and irrevocably guarantee payment to TGIIC of a sum not exceeding Rs. 	 on first written demand, without demur, for the due and faithful performance of the contract by the Bidder. This guarantee shall remain valid up to 	(contract period plus 3 months) and any claim thereunder must be made in writing before that date.

For 	Bank  Authorised Signatory  Date:

# Annexure VII — Financial Standing

Attested copies of audited financial statements (Profit & Loss Account, Balance Sheet), certified by a Chartered Accountant, for the last 5 financial years shall be furnished, along with a CA certificate confirming the average annual turnover and net worth of the firm.

| Financial Year | Annual Turnover (Rs. lakh) | Net Profit (Rs. lakh) |
| --- | --- | --- |

# Annexure VIII — Key Technical Personnel Proposed

| S.No. | Name | Designation / Role | Qualification | Relevant Experience (Years) |
| --- | --- | --- | --- | --- |

# Section XI — Fair Wages Clause

- The bidder shall pay not less than “fair wages” to all workmen/staff engaged by it in connection with the execution of this contract.

- “Fair wages” means wages, whether for time or piece work, notified by the appropriate Government authority from time to time for the category of work and area concerned, and in any case not less than the minimum wages prescribed under the Minimum Wages Act, 1948, as applicable in Telangana.

- The bidder shall notify the wages payable to each category of its workers, and any changes therein, and shall display the same at a prominent place at the site/location of work.

- TGIIC shall have the right to call for wage records and other statutory registers to satisfy itself that fair wages are being paid, and to deduct from the bidder's payments a suitable amount to make good any loss suffered by a worker on account of non-payment of fair wages, if established.

- The bidder shall comply with all applicable labour laws, including the Employees' Provident Funds Act, the Employees' State Insurance Act, and the Payment of Bonus Act, in respect of all workers engaged for this contract.

# Bidder's Acceptance

I/We have carefully read and understood the entire tender document, including the Scope of Work, Bill of Quantities, Technical Specifications, Service Level Agreement, General and Special Conditions of Contract, and all Annexures, and hereby submit our unconditional offer to execute the work described therein at the prices quoted in Section IV, and agree to keep this offer valid for 90 days from the date of opening of the technical bid.

For and on behalf of the Bidder

Name: 		Designation:

Signature & Company Seal: 		Date: